SEIS25D_GQG_v2

User Manual

2.5-D frequency-domain seismic modelling and full-waveform inversion

Consolidated standalone manual | 4 August 2026

S. Rouxel | Khalifa University of Science and Technology

# Contents

This manual is designed to remain useful when the complete code folder is shared outside GitHub. It consolidates the user-facing Markdown documentation supplied with the repository.

- Project overview and quick start

- Workstation workflow

- HPC workflow

- Input files

- Output files

# 1. Project overview and quick start

SEIS25D_GQG_V2 is a Fortran research code for 2.5-D frequency-domain seismic forward modelling, Fréchet-derivative calculation, and full-waveform inversion (FWI) in elastic and viscoelastic anisotropic media.

## FWI workflow

The frequency-domain wave equations are discretised on GQG grids. For each selected frequency, the code computes the modelled data and explicit Fréchet derivatives, which are used to form the gradient. The objective function is the L2 norm. L-BFGS uses the gradient and its limited update history to construct the model-update direction, while a More–Thuente line search selects an acceptable step length. The model is then updated, and the process is repeated across iterations and frequency bands until the configured stopping criteria are reached.

The repository currently provides a small (200 m × 200 m) three-component tilted transverse isotropy (TTI) example that can be compiled and run on a Linux workstation or on Windows through Ubuntu in Windows Subsystem for Linux (WSL). A larger Marmousi_VSP example is also available, with curated final HPC results for all 11 parameters displayed in the VTI composite figure.

## Capabilities

2D/2.5D frequency-domain forward modeling for elastic, viscoelastic, and viscoelastic TTI media;

2D/2.5D frequency-domain forward FWI for elastic, viscoelastic and viscoelastic TTI media

## Supported workstation platform

The tested workstation route is native Linux or Ubuntu under WSL, using the Intel oneAPI Fortran, MPI and oneMKL toolchain. Larger examples can be run through the supplied Slurm workflow. Sections 2 and 3 contain the dependencies, build options, resource requirements and troubleshooting guidance for each route.

## Get the complete code

The repository must be downloaded as one complete folder because the source, launchers, examples, and documentation use the directory structure shown below. Do not download individual Fortran or input files separately.

To download without Git:

- Open the project page on GitHub.

- Select Code, then Download ZIP.

- Extract the ZIP file to a location with sufficient free disk space.

- Open the extracted top-level folder—the repository root—which contains

README.md together with src, scripts, examples, and docs.

Windows users should keep the complete extracted folder together and access it from Ubuntu/WSL. For example:

C:\Users\path\SEIS25D_GQG_V2

is available inside WSL as:

/mnt/c/Users/path/SEIS25D_GQG_V2

Users familiar with Git may instead clone the complete repository and run the same commands from the cloned repository root.

## Quick start

In a Linux or Ubuntu/WSL terminal, initialize Intel oneAPI:

source /opt/intel/oneapi/setvars.sh

Move to the downloaded repository, then compile a release build without MUMPS:

make -C src CONFIG=release USE_MUMPS=0

Run the bundled Small TTI example:

./scripts/run_linux.sh

Section 2 explains executable permissions, debug builds, input settings, monitoring and troubleshooting.

## Repository layout

: src/                 Fortran source and Makefile
scripts/             Workstation launcher and cluster examples, plotting
examples/Small_TTI/  Small runnable TTI input set
docs/                Build, running, and input documentation
bin/                 Compiled executable (generated)
build/               Objects and module files (generated)
runs/                Run outputs and logs (generated, not committed)
tests/               Curated HPC test outputs, diagnostics, sanitized logs and figures

## References

Yang, Q., Zhou, B., Engsig, M., Won, M., Riahi, M. K., Al‐khaleel, M., & Greenhalgh, S. (2023). Numerical Fréchet derivatives of the displacement tensor for 2.5‐D frequency‐domain seismic full‐waveform inversion in viscoelastic TTI media. Near Surface Geophysics, 21(6), 395–413. https://doi.org/10.1002/nsg.12265

Zhou, B., Greenhalgh, S., & Maurer, H. (2012). 2.5-D frequency-domain seismic wave modeling in heterogeneous, anisotropic media using a Gaussian quadrature grid technique. Computers & Geosciences, 39, 18–33. https://doi.org/10.1016/j.cageo.2011.06.005

Preliminary results produced with this code were presented at the AGU25 Annual Meeting. This repository is being prepared to accompany the first full publication of the software and methodology.

Rouxel, S., and Bouchaala, F. (2025). Advancing 2.5D Frequency-Domain Full Waveform Inversion in Viscoelastic Anisotropic Media for VSP Applications. AGU25 Annual Meeting, presentation S33B-0258, abstract 1940271. AGU abstract (https://agu.confex.com/agu/agu25/meetingapp.cgi/Paper/1940271)

## License

This project is distributed under the GNU General Public License version 3 (LICENSE). The preferred scientific citation will be added before the first public release.

# 2. Workstation workflow

This chapter gives the complete Small TTI workstation workflow for native Linux or Ubuntu under WSL. It uses one MPI rank and Intel oneMKL PARDISO; MUMPS is not required. Small TTI requires at least 10GB of available RAM, though it has been successfully run up to 22Hz on a Lenovo Legion 5 16Gb. The much larger Marmousi_VSP case requires over 170GB of available RAM; use the resource guidance and Slurm workflow in Section 3.

## Install the required tools

Windows users must first install WSL 2 and Ubuntu. Follow the official Microsoft guide: https://learn.microsoft.com/en-us/windows/wsl/install. Native Linux users can skip this step.

A supplementary illustrated Windows 11 walkthrough is available at https://contabo.com/blog/how-to-install-wsl2-on-windows-11/. From an Administrator PowerShell window, the normal command is wsl --install. Restart Windows when requested, then open Ubuntu and create the initial Linux username and password.

Inside Ubuntu, install the standard build utilities with: sudo apt update, followed by sudo apt install -y build-essential make.

Install Intel oneAPI inside Linux/WSL using Intel's Linux installation guide: https://www.intel.com/content/www/us/en/docs/oneapi-toolkit/installation-guide-linux/latest/install-oneapi-toolkit-with-installer.html. Allow approximately 14 GB for a complete installation; the exact requirement depends on the release and selected components. If component selection is offered, install at least:

Intel Fortran Compiler, providing ifx: https://www.intel.com/content/www/us/en/developer/tools/oneapi/fortran-compiler-download.html;

Intel oneAPI Math Kernel Library (oneMKL): https://www.intel.com/content/www/us/en/developer/tools/oneapi/onemkl-download.html;

Intel oneAPI Threading Building Blocks (oneTBB): https://www.intel.com/content/www/us/en/developer/tools/oneapi/onetbb-download.html;

Intel oneAPI Collective Communications Library (oneCCL): https://www.intel.com/content/www/us/en/developer/tools/oneapi/oneccl-download.html;

Intel MPI Library, providing mpiifx and mpirun: https://www.intel.com/content/www/us/en/developer/tools/oneapi/mpi-library-download.html.

The documented workflow uses this Intel oneAPI toolchain because it is the configuration currently tested. Other compiler and library combinations are not yet supported.

Native Windows compilation is not supported. Windows users should build and run from Ubuntu in WSL.

## 2. Open Linux and initialize oneAPI

On Windows, open the standalone Ubuntu application or a WSL terminal. A repository downloaded at:

C:\Users\path\SEIS25D_GQG_V2

is visible inside WSL at:

/mnt/c/Users/path/SEIS25D_GQG_V2

These paths refer to the same files. Initialize Intel oneAPI after opening each new Linux terminal:

source /opt/intel/oneapi/setvars.sh

This command may be run from any directory. Confirm that the required commands are available:

## 3. Build

Move to the repository root, then build the normal optimized executable:

make -C src CONFIG=release USE_MUMPS=0

The executable is written to bin/Seis2D_GQG_FWI. Objects and generated Fortran module files are written under build/.

For troubleshooting source changes, use the slower debug configuration:

make -C src clean
make -C src CONFIG=debug USE_MUMPS=0

The debug configuration enables compiler warnings, runtime checks, floating-point traps and tracebacks but is much slower. Clean before switching between debug and release so that objects from different configurations are not mixed.

The available build settings are:

| Option | Values | Default | Meaning |
| --- | --- | --- | --- |
| CONFIG | release, debug | release | Optimized or diagnostic build |
| GF_STORAGE | sp, dp | sp | Green's-function storage precision |
| USE_MUMPS | 0, 1 | 0 | Compile optional MUMPS support |
| FC | compiler path/name | mpiifx | MPI Fortran compiler wrapper |

MUMPS has not been validated for the workstation workflow. Its optional HPC configuration is documented in the HPC guide (HPC.md).

## 4. Run the Small TTI example

Compilation and execution are separate. The launcher runs the executable currently in bin/Seis2D_GQG_FWI and does not rebuild it.

From the repository root:

./scripts/run_linux.sh

If execution permission was lost when downloading or copying the repository:

chmod +x scripts/run_linux.sh

The launcher:

locates the repository without a user-specific path;

uses the files in examples/Small_TTI;

fixes MPI to one rank;

detects the logical CPUs visible to Linux;

sets the OpenMP, oneMKL and program thread limits consistently;

detects total and currently available Linux/WSL memory;

warns, but does not refuse to run, when available memory is low;

creates a timestamped result directory;

records normal output, errors, the execution environment, and the complete selected MainInput configuration separately.

The numerical solver is selected in MainInput.inp, not in the launcher.

## 5. Launcher inputs

The launcher requires up to five positional file roles, and their valid combinations are defined in Section 4. MainInput.inp provides the modeling/inversion job configuration; the *Input*.inp contains the model parameter values. The launcher preserves those positions, including explicit empty arguments for unused intermediate roles.

The bundled examples use:

MainInput.inp
InitInput.inp
InitInputQ.inp
TrueInput.inp
TrueInputQ.inp

To use different filenames, edit the visible input block near the top of scripts/run_linux.sh:

INPUT_DIR="${PROJ_ROOT}/examples/Small_TTI"
MainInput="${INPUT_DIR}/MainInput.inp"
InitInput="${INPUT_DIR}/InitInput.inp"

Unused positional roles must be assigned an explicit empty string so that later file roles do not shift.

For a synthetic elastic inversion without Q files, keep all five roles but leave the unused Q roles empty:

InitInputQ=""
TrueInput="${INPUT_DIR}/TrueInput.inp"
TrueInputQ=""

See Section 4 Input files (INPUT_FORMATS.md) for the complete formats and roles

## 6. CPU and memory settings

By default, the launcher uses all logical CPUs visible to Linux or WSL. A lower thread count can be requested for testing:

OMP_NUM_THREADS=8 ./scripts/run_linux.sh

Do not request more threads than the CPUs available to WSL; oversubscription can substantially reduce performance. MPI remains fixed to one workstation rank.

The launcher exports currently available memory to the program as MEM_MAX and records it in environment.txt.

## 7. Results and logs

Each run creates a directory such as:

runs/Small_TTI/output_YYYYMMDD_HHMMSS/

Section 5 describes the launcher records, configuration_summary.txt, and scientific output files. An empty or nearly empty error.log is normal; after an abnormal exit, inspect both logs.

## 8. Protect long runs with tmux

Run long inversions from the standalone Ubuntu terminal inside tmux so that a VS Code or terminal disconnection does not stop the process:

tmux new -s seis2d
./scripts/run_linux.sh

Detach by pressing Ctrl+B, releasing both keys, and then pressing D. Reconnect later with:

tmux attach -t seis2d

## 9. Common build problem

If mpiifx or mpirun is not found, initialize oneAPI in the current terminal:

source /opt/intel/oneapi/setvars.sh

Building under /mnt/c is supported and convenient. Keeping the repository in WSL's native Linux filesystem may improve compilation and I/O performance, but is not required for Small TTI.

# 3. HPC workflow

This workflow is intended for Marmousi_VSP and other cases that exceed ordinary workstation memory. The provided Marmousi_VSP example requires over 170 GB of available RAM; the supplied six-node configuration targets completion within the 48-hour production limit on the reference cluster. A high-memory workstation may also run the case, but fewer CPU cores will make the explicit Fréchet calculations slower. On any HPC or cloud system, verify the memory actually available to the job and retain operating-system and numerical-library headroom. Cluster module names, account, partition, CPU allocation, and time limits must be adapted locally.

Before relying on a new cluster, perform a short Small TTI build/run first by editing the visible input block in run_slurm.sh. Confirm module compatibility, thread placement, memory reporting, Slurm integration, and numerical output before allocating resources to Marmousi.

## Files

## 1. Load the cluster toolchain

List the modules available on your system, then load the Intel compiler, Intel MPI, and oneMKL modules. For example only:

module purge
module load intel
module load intel-mpi
module load mkl
# module load mumps  # uncomment only when building with USE_MUMPS=1

The exact names must be replaced with those used by the target cluster.

## Build

src/Makefile.hpc supplies visible HPC build defaults and includes the

common src/Makefile so the source list and dependencies remain identical.

### Release build

From the repository root, build the optimized version used for normal production runs:

make -C src -f Makefile.hpc CONFIG=release

release is the default, so omitting CONFIG=release produces the same build:

make -C src -f Makefile.hpc

The default build uses PARDISO, matching the published Marmousi input. The executable is written to bin/Seis2D_GQG_FWI.

### Debug build

For troubleshooting, compile with runtime checks and debugging information:

make -C src -f Makefile.hpc CONFIG=debug

Debug builds are slower and should not be used for normal Marmousi production runs. Before changing between release and debug modes, remove the objects from the previous build and then rebuild:

make -C src -f Makefile.hpc clean
make -C src -f Makefile.hpc CONFIG=debug

The Slurm launcher does not compile the program or choose a build mode. It runs the executable currently present in bin/Seis2D_GQG_FWI, so compile the intended configuration before submitting the job.

### Optional MUMPS build

To try MUMPS, load the cluster's MUMPS module and build with:

make -C src -f Makefile.hpc USE_MUMPS=1

MUMPS can also be combined with a debug build:

make -C src -f Makefile.hpc CONFIG=debug USE_MUMPS=1

The solver must then be selected in MainInput.inp. The MUMPS module must expose its headers and libraries. If it does not define MUMPS_DIR, pass the installation root to make, and override MUMPS_LIBS when the cluster uses different dependency or library names. The editable settings and link pathway are shown at the top of src/Makefile.hpc.

## 3. Configure Slurm

Open scripts/run_slurm.sh and edit the first two settings sections:

uncomment and set --partition and --account when required;

choose a time limit accepted by the cluster;

retain six nodes for the published Marmousi runtime target, or adapt after

benchmarking on the destination cluster;

set --cpus-per-task to the physical/logical CPUs assigned to each rank;

ensure only 1 MPI task per node

request a node with enough memory for Marmousi;

replace the generic module names with the local names.

The example requests all available memory on each allocated node with --mem=0; some clusters require an explicit per-node value instead.

### Other datasets and filenames

The Slurm launcher keeps the five input roles together in one visible block. Edit INPUT_DIR and the required filenames directly.

Optional roles may be assigned empty strings while their positional slots remain intact.

The supplied launcher selects MainInput_C33.inp. To run another published Marmousi inversion, change the visible MainInput filename. See Input files (INPUT_FORMATS.md) for the required contents, units, indexing, and supported combinations of these files.

## 4. Submit

From the repository root:

sbatch scripts/run_slurm.sh

The script locates the repository automatically and selects examples/Marmousi_VSP/MainInput_C33.inp. Results are written to:

runs/Marmousi_VSP/slurm_JOB_ID/

The result directory contains environment.txt, configuration_summary.txt, and all program outputs. The configuration summary records the selected MainInput filename, SHA-256 checksum, and complete contents. Slurm stdout and stderr are copied into the directory every five minutes and again when the job exits.

# 4. Launcher Input files

This document describes the input files accepted by SEIS25D_GQG_V2 and how they fit together. It is based on the current Fortran reader and the bundled examples/Small_TTI example.

## Files passed to the executable

The executable currently uses five positional file slots:

| Position | Role | When required |
| --- | --- | --- |
| 1 | MainInput job configuration | Always |
| 2 | Working elastic model (InitInput) | Always |
| 3 | Working attenuation model (InitInputQ) | Viscoelastic runs only |
| 4 | True/reference elastic model (TrueInput) | Synthetic inversion |
| 5 | True/reference attenuation model (TrueInputQ) | Synthetic viscoelastic inversion |

For forward modeling, position 2 contains the model to simulate even though its historical name is InitInput. For pure elastic work, positions 3 and 5 are unused. For a real-data viscoelastic inversion, positions 4 and 5 are unused.

The interface is positional. If an unused slot occurs before a used one, pass an empty quoted argument ("") so that later files do not shift into the wrong roles. An InitInputQ without an elastic InitInput is not a valid combination.

The supplied Linux launcher contains the recommended argument combinations.

## MainInput.inp

MainInput.inp controls the job, frequencies, physics, solver, optimization, grid, geometry, and acquisition pattern. Comment-like heading lines are part of the current ordered format: retain the structure and change the values beneath the headings.

Current transition:

Some parameters (Topography, acquisition geometry) can be supplied embedded and/or read from external files; however, the provided examples use exclusively the fully embedded format for maximum portability.

### Job and mode

--JOB AND MODE CONTROL INV -- I25D--
1 1

The first integer is INV:

| Value | Operation |
| --- | --- |
| 0 | Synthetic forward modeling |
| 1 | Synthetic full-waveform inversion |
| 2 | Inversion of externally supplied observations |

I25D selects the 2.5-D formulation (1) or its alternative pure 2D mode (0). Use the bundled value unless you understand the associated modeling change.

### Frequency bands and observed data

--FREQUENCY BANDS--
2
3 7.0 12.0 22.0

2 37.0 50.0
--FREQUENCY DATA FILES--
-

The first value is the number of frequency bands. Each following band line starts with the number of frequencies in that band, followed by those frequencies in hertz.

A single - indicates that no external spectra are supplied and requests internally generated synthetic data.

### External spectral data

For external observations, supply one spectral-data filename per frequency. The filename order must match the frequency order.

--FREQUENCY BANDS--
2
3 7.0 12.0 22.0

2 37.0 50.0
--FREQUENCY DATA FILES--
\path\data_7hz.txt

..
path\data_50zh.txt

External spectral data file format is

Freq ND Real Imag
10.0 1 10.94 13.05

…
10.0 ND 11.20 12.74

Each row contains frequency, acquisition-row index, and the real and imaginary parts of the complex spectrum. The row order and count must match the selected acquisition pattern. A short text header is accepted.

### Physics and inverted parameters

--ANISOTROPIC MODEL (IANISO ITHOM IVISCO CMIN CMAX)--
7 0 1 1.0 2.5
'rho', 'c11', 'c13', 'c33', 'c44', 'c66', 'the', 'Q11', ...
 0 0 0 1 0 0 0 0 0 0 0 0

IANISO determines the elastic parameterization: TTI (7), VTI (6), or isotropic (3). The Small TTI example uses the seven elastic fields rho, C11, C13, C33, C44, C66, theta. Marmousi is VTI and uses six elastic fields: rho, C11, C13, C33, C44, C66.

ITHOM enables (1) or disables (0) Thomsen-parameter parametrisation (rho, Vp, Vs, epsilon, delta, gamma). Inversion is not yet available in that parametrisation when IVISCO is enabled;

IVISCO enables (1) or disables (0) attenuation.

CMIN and CMAX are the minimum expected Vs and the maximum expected Vp used by the preliminary grid checks.

The next line names the elastic and Q parameters in the chosen parametrisation.

The following integers assign inversion groups: 0 keeps a parameter fixed; a positive integer assigns it to an inversion group/stage. The number of entries must match the selected parameterization. You may use the same order throughout the inversion or, when multiple frequency bands are active, provide one tailored line per frequency band. Example for two frequency bands:

For the first frequency band, only C33 is inverted; then in the second frequency band C33 and C44 are jointly inverted until convergence, then Q33 is inverted.

'rho', 'c11', 'c13', 'c33', 'c44', 'c66', 'the', 'Q11', ...
0 0 0 1 0 0 0 0 0 0 0 0
0 0 0 1 1 0 0 0 0 2 0 0

### Solver and inversion controls

--SOLVER CHOICE--
2
--REGULARIZATION (REG lambda)--
0 0.0
--LBFGS SETTINGS (MML LBFGS_TYPE)--
5 0
--OPTIMIZATION CONTROLS (MAXITER cost_conv_tol misfit_conv_tol)--
10 1e-3 1e-4
--PRECONDITIONING (active, beta value)--
0 0.001
--GRADIENT TAPER AND SMOOTHING (active, Z1, Z2, sigmax, sigmaz) --
0 0.0 0.0 0 0

Solver choices are:

| Value | Solver |
| --- | --- |
| 1 | LU legacy solver |
| 2 | Intel oneMKL PARDISO |
| 3 | MUMPS |

A build made without MUMPS will reject solver 3 with an explicit message.

The remaining lines control regularization (not fully implemented; keep at (0,0), L-BFGS history/type, maximum iterations and convergence tolerances, preconditioning strength, and gradient tapering/smoothing. Begin with the supplied example values.

### GQG Grid parameters

--GLOBAL GRID (NORD DX DZ)--
3 5.0 5.0

NORD: spatial interpolation/order for that band, typically 3 or 5;

DX: horizontal GQG subdomain size in metres;

DZ: horizontal GQG subdomain size in metres.

- The program will check at input that the provided combination fulfils the preliminary 5 points-per-wavelength and more than 2 subdomains per wavelength criteria based on CMIN/CMAX. A second definitive check is performed using the full parameter after GQG interpolation.

#### Frequency-dependent grids (optional)

For a multiband run, each frequency band may use its own grid order and sampling. Place the optional block immediately after the global grid line if using:

--GLOBAL GRID (NORD DX DZ)--
NORD(NFBAND) DX(NFBAND) DZ(NFBAND)
--Band meshing
NORD(1) DX(1) DZ(1)
NORD(2) DX(2) DZ(2)
...
NORD(NFBAND) DX(NFBAND) DZ(NFBAND)

The heading must contain the case-sensitive text Band meshing. Exactly one value row is required for each frequency band, in the same order as the bands listed under --FREQUENCY BANDS--. Each row contains:

For example, a two-band inversion can use:

--GLOBAL GRID (NORD DX DZ)--
5 50.0 50.0
--Band meshing
5 60.0 60.0
5 50.0 50.0

The first row applies to frequency band 1 and the second to band 2. This makes it possible to use a coarser, less expensive grid for lower frequencies and a finer grid for higher frequencies.

The global NORD DX DZ line remains mandatory even when this optional block is present and should correspond to the finest desired grid. Every selected grid is checked against CMIN, CMAX, and the frequency range of its band before modeling begins.

### Grid block

The grid currently follows the global-grid settings inside MainInput.inp:

MX
MZ(1) MZ(2) ... MZ(MX)
--GRID COORDINATES--
x(1) z(1,1) ... z(1,MZ(1))
x(2) z(2,1) ... z(2,MZ(2))
...

MX is the number of horizontal grid columns. MZ(i) is the number of vertical points in column i. Each coordinate row begins with its horizontal coordinate and contains that column's vertical coordinates. Coordinates are in metres; the example uses negative depth below z = 0. A row can be of variable length if the topography is not regular.

Every elastic and Q model must use the same MX, MZ, and row ordering. grid_file.txt in the example is currently a reference copy, not a separate command-line input.

### Topography

MainInput.inp first supplies the topography size and surface flag:

NTO IS0

This is followed either by NTO coordinate rows embedded directly in the main input, or by the path to an external topography file. The external file repeats the header:

NTO IS0
x(1) z(1)
...
x(NTO) z(NTO)

NTO is the number of surface points, followed by NTO x/z coordinate pairs in metres. IS0 is the free-surface (0) / top-absorbing-boundary (1) flag.

### Source and receiver geometry

The main input defines the total number of source/receiver points (NSR), the number of sources (NSS), and a legacy geometry-mode value.

The bundled example uses the compact regular-geometry generator embedded in MainInput.inp. All coordinates are in metres.

### Regular source/receiver generator

The compact generator is selected automatically when the first non-blank line after the NSR/NSS/ISR90 line is a short numeric component-setting line. No separate mode switch is required. It assumes a flat topography. Its block is:

NSR  NSS  ISR90
NCOMPS  NCOMPR
source_x0  source_dx  source_z0  source_dz  NSS
top_x0     top_dx     top_z0     top_dz     N_top
left_x0    left_dx    left_z0    left_dz    N_left
right_x0   right_dx   right_z0   right_dz   N_right
bottom_x0  bottom_dx  bottom_z0  bottom_dz  N_bottom

NCOMPS and NCOMPR are the numbers of source and receiver components (1 to 3). ISR90 is a retained legacy setting keep at 0.

For each row, x0 and z0 are the first point, dx and dz are the interval increments, and N is the number of points. Point j is placed at x = x0 + (j - 1) dx and z = z0 + (j - 1) dz; all generated y coordinates are zero. Set N to 0 to omit a receiver side.

The generated array is ordered as all sources, followed by top, left, right, and bottom receivers. The program calculates NRR = N_top + N_left + N_right + N_bottom and then recomputes NSR = NSS + NRR. Keep the NSR value written on the first line consistent with this total for clarity. Every generated coordinate must lie inside the model bounds.

The Small TTI example uses:

50 10 0
3 3 0.0
10  20   -5    0   10
0   20    0    0   11
0    0  -20  -20    9
200  0  -20  -20    9
0   20 -200    0   11

For irregular spacing/ non-flat topography and real-acquisition location, source and receiver information must be individually listed either in embedded form or through an external file.  The first NSS geometry row lists all sources, and the remaining NSR - NSS rows are receivers with the format below.

x y z ICSR comp1_x comp1_y comp1_z [comp2_x comp2_y comp2_z ...]

ICSR is the number of components (1 to 3), followed by that many direction vectors. Component conventions are X, Y, Z; a one-component case corresponds to a vertical case, while two components are X and Z.
Example for 1 conventional seismic source, and one 3C receiver:

x y z 1 0 0 1 !source

x y z 3 1 0 0 0 1 0 0 0 1 ! receiver

### Acquisition pattern

After the grid, the data-acquisition section contains an optional acquisition-based inversion weight file path (- for none), followed by the pattern selection.

A value of 1 generates all possible source/receiver/component pairings. Alternatively, the list can be embedded or supplied in an external acq_pattern.txt file:

ND
row_id source_id source_component receiver_id receiver_component
...

The row ID may be omitted, leaving four integers per row. Component IDs are 1 = X, 2 = Y, and 3 = Z. Source IDs run from 1 to NSS. Receiver IDs use the global geometry numbering, from NSS + 1 through NSR. ND is the total number of listed source/receiver/component pairings.

## Model .inp files

### Elastic model files

InitInput and TrueInput contain one block per elastic parameter, in the exact order selected by IANISO and ITHOM. Each block has a label and MX data rows:

--PARAMETER NAME--
x(1) value(1,1) ... value(1,MZ(1))
...
x(MX) value(MX,1) ... value(MX,MZ(MX))

For the bundled TTI case, the blocks are rho, C11, C13, C33, C44, C66, and theta. Density is in kg/m³, stiffness values are supplied in GPa, and angle is in degrees. InitInput is the working/starting model; TrueInput is the reference model used to generate synthetic observations.

For (visco) elastic cases, parameters are rho, Vp, Vs (Qp, Qs) in km/s.

For VTI cases parameters Thomsen parametrization is also possible with rho, Vp, Vs (in km/s)

and epsilon, delta, gamma, and their associated Qs; however, (visco VTI) FWI is not currently available in that parametrization.

### Attenuation model files

InitInputQ and TrueInputQ use the same grid-shaped block format. The Small TTI case contains Q11, Q13, Q33, Q44, and Q66, which are dimensionless. These files are used only when IVISCO = 1.

# 5. Output files

This chapter describes the launcher records and scientific files produced by SEIS25D_GQG_V2. The exact set depends on the run mode, active parameters, frequencies, iterations, preprocessing and diagnostic-output settings; forward modelling therefore produces fewer files than a full inversion.

## Plotting scripts

The publication package provides two equivalent plotting scripts under scripts/ for displaying the principal model, gradient and spectral-data outputs described below:

scripts/seis25d_plot.py for Python;

scripts/seis25d_plot.m for MATLAB.

. Both scripts keep the case-dependent plotting settings near the top of the file so that users can readily adjust paths, parameter names, well position, and plotting-depth limits. These scripts are intended for common QC; specialist diagnostic files and publication quality may require additional plotting code.

## Result directory

The launchers create a new result directory before starting the program, so the program's relative output filenames are collected in one place.

Workstation example:

runs/Small_TTI/output_YYYYMMDD_HHMMSS/

Slurm example:

runs/Marmousi_VSP/slurm_JOB_ID/

## Launcher records

These files are created by the launchers rather than by the Fortran program.

| File | Purpose |
| --- | --- |
| run.log/ slurm-JOB_ID.out | Standard program output, progress messages and timings/ Slurm scheduler standard output, copied into the HPC result directory |
| error.log/ slurm-JOB_ID.err | Standard error and available out-of-memory evidence/ Slurm scheduler standard error, copied into the HPC result directory |
| environment.txt / configuration_summary.txt | Execution environment and hardware information / selected MainInput filename, SHA-256 checksum and complete contents. |

An empty or nearly empty error.log is normal. For a failed run, inspect the end of both run.log and error.log. On HPC, also inspect the original Slurm logs in the submission directory if copying was interrupted. Additional run.log writes and/or QC files can be turned on by activating DEBUG_OUTPUT to .TRUE. in targeting subroutines .

## Geometry and model-grid exports

| Pattern | Meaning |
| --- | --- |
| m_Grid.dat | Numbered GQG-grid point coordinates |
| Seis2D_GQG.txt | Legacy coordinate export retained by the current program, include interpolation information for source and receivers |
| m_Top.dat | Topography and related plotting coordinates |
| m_SR.dat | Source/receiver geometry prepared for plotting and QC |

m_Grid.dat and Seis2D_GQG.txt contain a point index followed by the two model coordinates. Geometry coordinates use the same coordinate convention and units as the corresponding input files.

## Gridded field format

Model, gradient, updated-model, Green-function and many Fréchet files use a plain-text rectangular grid designed for direct plotting:

NZ  z(1)  z(2) ... z(NZ)
x(1)  value(1,1) ... value(1,NZ)
x(2)  value(2,1) ... value(2,NZ)
...

The first number on the first row is the number of vertical samples. The remaining values on that row are the vertical coordinates. Each later row starts with one horizontal coordinate followed by the field values at all vertical coordinates. Some interpolation routines represent air cells with a large sentinel value (currently approximately 1.0E20); plotting scripts should mask these values rather than display them as physical properties.

The values retain the physical or derived units of the named parameter. Model exports are not automatically converted to plotting units such as GPa or g/cm³.

## Initial and reference models

| Pattern | Meaning |
| --- | --- |
| mI_{PARAM}.dat | Initial / working model for PARAM |
| mT_{PARAM}.dat | True/reference model for a synthetic case |

PARAM is the internal parameter label, for example C33, rho, the or a Q parameter. Only parameters available in the selected physical model are written. A real-data run without a true/reference model will not produce the corresponding mT_* files.

## Spectral data and calculated responses

### Trace files

GT0_IFQ_ITER.txt contains the observed or synthetic-reference complex spectrum. GO_IFQ_ITER.txt contains the response calculated from the current model. IFQ is the one-based frequency index and ITER is the iteration number; reference data are commonly written with iteration zero.

Each row has 14 columns:

| Column | Meaning |
| --- | --- |
| 1 | Frequency index IFQ |
| 2 | Iteration |
| 3 | Data index |
| 4 | Source index |
| 5 | Source component index |
| 6 | Receiver index |
| 7 | Receiver component index |
| 8–9 | Source X and Z coordinates |
| 10–11 | Receiver X and Z coordinates |
| 12–13 | Real and imaginary spectral values |
| 14 | Complex amplitude/magnitude |

### Forward-model export

For forward-only runs, OBS_FREQ_FREQUENCY.txt contains:

frequency  data_index  real_part  imaginary_part

Despite the historical OBS prefix, this is the spectral dataset exported by the forward-only path and can be used as external spectral input for an inversion job.

## Residual and inversion histories

### out_resid.txt

When acquisition geometry is available, each row contains:

iteration  data_index  source_index  source_component
receiver_index  receiver_component  source_x  source_z
receiver_x  receiver_z  residual_real  residual_imag  acquisition_weight

Rows are appended during the run. Their order follows the internal data mapping.

### out_diag.txt

List the following parameters to monitor inversion performance. One row is written for each active parameter and iteration:

frequency  iteration  previous_cost  scaled_gradient_norm  residual_RMS
iteration_seconds  frequency_band  frequency_index  parameter_order
parameter_index  active_parameter_index  global_iteration

### out_linesearch.txt

List line search diagnostic history. The header written by the program defines:

FREQ  ITER  trial  TAG  ALPHA  trial_cost  reference_cost

### out_lbfgs_hist.txt

This is the detailed L-BFGS diagnostic history. Its header names the gradient, direction, step and curvature quantities, accepted/skipped history columns, and diagnostic reason codes.

## Gradient files

Gradient files are generated for each inverted parameter, frequency and iteration:

| Pattern | Meaning |
| --- | --- |
| GRADr_{PARAM}_FREQUENCY_ITNN.dat | Masked physical/data gradient in parameter-scaled model space |
| GRADrsc_{PARAM}_FREQUENCY_ITNN.dat | Scaled and parameter-balanced gradient |
| GRADsm_{PARAM}_FREQUENCY_ITNN.dat | Smoothed (preconditioned) gradient, when smoothing is enabled |
| GRADrpm_{PARAM}_FREQUENCY_ITNN.dat | Preconditioned gradient, when preconditioning is enabled |

GRADrpm is intentionally absent when preconditioning is disabled. The Small TTI configuration used during workstation validation therefore produced GRADr and GRADrsc but no GRADrpm; but they are enable in Marmousi_VSP.

## Updated model files

The current historical naming pattern is:

PCBB_{{PARAM}_FREQUENCY_ITNN}.dat
PCBA_{{PARAM}_FREQUENCY_ITNN}.dat

PCBB is written from the current accepted model state.

PCBA is an additional updated/trial-model diagnostic written only when the corresponding debug output is enabled.

## Fréchet-derivative files

When detailed Fréchet export is enabled, filenames include the data/source index, parameter, and frequency. Current prefixes include:

F_RE...
F_IM...
F_RE_SRC1_SUM...
F_IM_SRC1_SUM...
F_ABS_SRC1_SUM...
F_RMS_SRC1_SUM...

These are diagnostic fields for the real part, imaginary part, and selected source summaries. They can be numerous and very large. Their absence does not mean that Fréchet derivatives were not calculated internally; file export is a separate diagnostic choice.

## Green-function files

Optional wavefield exports use real/imaginary pairs with names containing the field prefix, source/component indices and frequency, for example patterns of the form:

PREFIX_ReG{SC}_FREQUENCY.dat
PREFIX_ImG{SC}_FREQUENCY.dat

Green functions are normally held in memory for modelling and derivative calculation, only exported in single source runs.